# Haber–Bosch Process Simulation in DWSIM

1. Project Overview

I used the open-source process simulation software DWSIM to model an ammonia synthesis process based on the Haber–Bosch process. The project involved developing an equilibrium-based reaction model and selecting appropriate operating conditions

The goal was to develop experience in process simulation, reaction modelling and the application of chemical engineering principles to process design. Exposing myself to understand scale up of simple reactions

2. Simulation Overview

The overall ammonia synthesis reaction was modelled as:

N₂ + 3H₂ ⇌ 2NH₃

The process flowsheet consisted of a compressor, heater, equilibrium reactor, cooler and gas–liquid separator.

Process Units and Operating Conditions

| Process Unit | Operating Conditions |
| --- | --- |
| Compressor | Adiabatic operation; outlet pressure = 200 bar |
| Heater | Outlet temperature = 700 K; no pressure drop; 100% efficiency |
| Equilibrium Reactor | Isothermal operation at 700 K; no pressure drop |
| Cooler | Outlet temperature = 250 K |
| Gas–Liquid Separator | Adiabatic operation |
| Feed | 4mol/s 0.75 H₂ and 0.25 N₂ mole fractions, temperature = 250 , pressure = 30bar |

The process was modelled at high pressure to represent the high-pressure conditions associated with ammonia synthesis. Following the reactor, the stream was cooled before entering the gas–liquid separator to facilitate separation of the process products.

3. Engineering Decisions

Operating Pressure

A reactor pressure of 200 bar was selected. Increasing pressure favors ammonia formation because the reaction produces fewer moles of gas than are present in the reactants. Maintaining high pressure therefore supports ammonia formation according to Le Chatelier's principle.

The same pressure was maintained through the reactor in the simulation by specifying no pressure drop.

Reactor Temperature and Calculation Mode

The equilibrium reactor was configured to operate isothermally at 700 K. This ensured that the reactor temperature remained constant during the equilibrium calculation.

An isothermal calculation mode was selected rather than an adiabatic mode because the latter allows the reactor temperature to change as a result of the reaction and the absence of external heat added.

Reaction Modelling

The reaction was modelled using an equilibrium reactor. Nitrogen was selected as the base compound in the reaction model, Nitrogen is easier to store and acquire than hydrogen so I assumed that in actual chemical plants they will have more nitrogen than hydrogen making nitrogen in excess.

The reaction was modelled in the vapor phase, and the required thermodynamic data were entered into DWSIM.

4. Process Separation

After the equilibrium reactor, the stream was cooled to 250 K before entering the gas–liquid separator.

The cooling step was included to promote condensation and facilitate the subsequent separation of ammonia from the remaining gas-phase components. The separator was configured using adiabatic calculation mode.

5. Skills Developed

This project developed practical experience in:

Process simulation and flowsheet development

Reaction modelling using an equilibrium reactor

Selection and application of operating conditions

Thermodynamic modelling

Understanding the relationship between pressure, temperature and chemical equilibrium

Process separation and phase behavior

Application of Le Chatelier's principle to process design

Troubleshooting and analysing a process simulation

This simulation was at first hard to do and understanding getting into issues of whether to use a conversion reactor or equilibrium reactor. Understanding why compression and heating happen and why the reaction is isothermal instead of adiabatic. drawing from most of the things I learnt in first year to make sense and understand reaction modelling.

The ammonia was produced at 0.96mol/s with 99% purity. This conclusion makes sense as there is 1 mole of nitrogen coming in from the feed of 4mol/s 96% conversion of that nitrogen produces 0.96 mol of ammonia. The reaction was very efficient. I would next time add a recycle loop to increase the conversion closer to 99%.